HTTP/CGI Command Programing Guide

Revision Log

| Date | Version | Description / Major change |
| --- | --- | --- |
| 2007/09/18 | V1.0 | Create document |
| 2008/10/16 | V1.1 | Add motion detection |

1	HTTP/CGI COMMAND INDEX

Please refert to document:

- ipcam_http_api.pdf

- ipcam_http_param.pdf

2	SAMPLE COMMAND

2.1	System Category

2.1.1	    Get brand Info

Example:

http://<servername>/cgi-bin/view/param?action=list&group=General.Brand

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| Brand | String | 7/0 | The brand of the product. |
| ProdFullName | String | 7/0 | The full name of the product. |
| ProdNbr | String | 7/0 | The product number. |
| ProdShortName | String | 7/0 | The short name of the product. |
| ProdType | String | 7/0 | The product type. |
| WebURL | String | 7/0 | The URL to visit for support and information about the product. |

root.General.Brand.Brand=XXXX

root.General.Brand.ProdFullName=XXXX CMOS Network Camera

root.General.Brand.ProdNbr=F12345

root.General.Brand.ProdShortName=F12345 CMOS Network Camera

root.General.Brand.ProdType=Network Camera

root.General.Brand.WebURL=www.xxxxx.com

2.1.2	    Get/Set account Info

Example: Get account information

http://<servername>/cgi-bin/view/param?action=list&group=General.UserID

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| ViewerAuthen | on/off | 1/6 | Enable/disable viewer authentication. |
| ViewerModeDefault | 000007ff, 0000063f, 0000003f | 1/6 | Security level. 000007ff for administrator, 0000063f for operator, 0000003f for viewer. |
| Admin | string | 1/6 | Based 64 encrypted account for Administrator. |
| User1~ User9 | String | 1/6 | Based 64 encrypted account and security level for user . |

root.General.UserID.ViewerAuthen=on

root.General.UserID.ViewerModeDefault=000007ff root.General.UserID.Admin=YWRtaW46YWRtaW4=

root.General.UserID.User1=MTExMToxMTEx,0000003f

root.General.UserID.User2=MjIyMjoyMjIy,0000063f

root.General.UserID.User3=MzMzMzozMzMz,000007ff

root.General.UserID.User4=Og==,000007ff

root.General.UserID.User5=Og==,000007ff

root.General.UserID.User6=Og==,000007ff

root.General.UserID.User7=Og==,000007ff

root.General.UserID.User8=Og==,000007ff

root.General.UserID.User9=Og==,000007ff

Example: Set account information

http://<servername>/cgi-bin/admin/param?action=update&General.UserID.ViewerAuthen=off&General.UserID.User1= MzMzMzozMzMz,000007ff

Return:

OK

2.1.3	    Get Firmware info

Example:

http://<servername>/cgi-bin/view/param?action=list&group=Properties.Firmware

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| BuildDate | string | 1/0 | Firmware build date. |
| Version | string | 1/0 | Firmware version |

root.Properties.Firmware.BuildDate=Tue Mar 20 22:26:10 CST 2007

root.Properties.Firmware.Version=LM.0.3.14

2.1.4	    Get hardware version

Example:

http://<servername>/cgi-bin/view/param?action=list&group=Properties.Hardware

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| Version | string | 1/0 | Hardware version |

root.Properties.Hardware.Version=00

2.1.5	    Get video support format

Example:

http://<servername>/cgi-bin/view/param?action=list&group=Properties.Image.Format

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| Format | string | 1/0 | Board support video format. (separated by “,”) |

root.Properties.Image.Format=mpeg4,mjpeg

2.1.6	    Get video support resolution

Example:

http://<servername>/cgi-bin/view/param?action=list&group=Properties.Image.Resolution

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| Resolution | string | 1/0 | Board support video resolution. (separated by “,”) |

root.Properties.Image.Resolution=176x120,352x240,720x480

2.1.7	    Get PTZ support info

Example:

http://<servername>/cgi-bin/view/param?action=list&group=Properties.PTZ.PTZ

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| PTZ | yes, no | 1/0 | Board support PTZ capability |

root.Properties.PTZ.PTZ=yes

2.1.8	    Get DIDO support info

Example:

http://<servername>/cgi-bin/view/param?action=list&group=Properties.DIDO

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| DI.DI | yes, no | 1/0 | Board support DI capability |
| DI.Type | virtual, physical | 1/0 | Board support DI type |
| DO.DO | yes, no | 1/0 | Board support DO capability |
| DO.Type | virtual, physical | 1/0 | Board support DO type |

root.Properties.DIDO.DI.DI=no

root.Properties.DIDO.DI.Type=virtual

root.Properties.DIDO.DO.DO=no

root.Properties.DIDO.DO.Type=virtual

2.1.9	    Get Motion Detection support info

Example:

http://<servername>/cgi-bin/view/param?action=list&group=Properties.Motion.Motion

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| Motion | yes, no | 1/0 | Board support Motion Detection capability |

root.Properties.Motion.Motion=yes

2.1.10	Get audio support info

Example:

http://<servername>/cgi-bin/view/param?action=list&group=Properties.Audio

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| Audio | yes, no | 1/0 | Board support Audio capability |
| Format | string | 1/0 | Board support Audio format |

root.Properties.Audio.Audio=yes

root.Properties.Audio.Format=g711

2.1.11	Get/Set Network Interface info

Example: Get Network interface information

http://<servername>/cgi-bin/view/param?action=list&group=General.Network.eth0

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| Broadcast | IP address | 6/6 | Broadcast address. (read only) |
| DNSAuto | on, off | 6/6 | Specifies if the DNS server should be obtained from a DHCP server. |
| DNSServer1 | IP address | 6/6 | Primary Domain Name System server. |
| DNSServer2 | IP address | 6/6 | Secondary Domain Name System server. |
| BootProto | dhcp, none | 6/6 | Enable/disable dynamic IP address assignment. |
| IPAddress | IP address | 6/6 | IP address. The physical address of the device on the network. |
| MACAddress | 00:xx:xx:xx:xx:xx | 6/4 | MAC address. The unique identity of the device. (read only) |
| SubnetMask | IP address | 6/6 | Subnet mask. Divides the network |
| DefaultRouter | IP address | 6/6 | Default gateway address |
| IPAddressRange | string | 6/6 | Auto assign range when DHCP fail. Ex:192.168.0.99-192.168.0.254 |

root.General.Network.eth0.Broadcast=192.168.1.255

root.General.Network.eth0.DNSAuto=192.168.1.255

root.General.Network.eth0.DNSServer1=192.168.1.255

root.General.Network.eth0.DNSServer2=192.168.1.255

root.General.Network.eth0.BootProto=192.168.1.255

root.General.Network.eth0.IPAddress=192.168.1.255

root.General.Network.eth0.MACAddress=192.168.1.255

root.General.Network.eth0.SubnetMask=192.168.1.255

root.General.Network.eth0.DefaultRouter=192.168.1.255

root.General.Network.eth0.IPAddressRange=192.168.1.255

Example: Set Network interface information

http://<servername>/cgi-bin/admin/param?action=update&General.Network.eth0.BootProto=off&General.Network.eth0.IPAddress=192.168.1.99

Return:

OK

2.1.12	Get/Set UPNP info

Example: Get UPnP information

http://<servername>/cgi-bin/view/param?action=list&group=General.Network.UPnP

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| Enabled | yes, no | 6/6 | Enables Universal Plug and Play. |
| NATTraversal.Enabled | yes, no | 6/6 | Enables/disables NAT traversal. |
| NATTraversal.HTTPPort | number | 6/6 | HTTP port number for NAT traversal. |
| NATTraversal.HTTPSPort | number | 6/6 | SSL port number for NAT traversal. |
| NATTraversal.HTTPViewerPort | number | 6/6 | MPEG4 port number for NAT traversal. |
| NATTraversal.HTTPViewerPortSSL | number | 6/6 | MPEG4 over SSL port number for NAT traversal |
| NATTraversal.HTTPViewerPort2 | number | 6/6 | MJPEG port number for NAT traversal |
| NATTraversal.HTTPViewerPort2SSL | number | 6/6 | MJPEG over SSL port number for NAT traversal |
| NATTraversal.HTTPViewerPort3 | number | 6/6 | RTSP – computer view port number for NAT traversal |
| NATTraversal.HTTPViewerPort4 | number | 6/6 | RTSP – mobile view port number for NAT traversal |

root.General.Network.UPnP.Enabled=yes

root.General.Network.UPnP.NATTraversal.Enabled=no

root.General.Network.UPnP.NATTraversal.HTTPPort=80

root.General.Network.UPnP.NATTraversal.HTTPSPort=443

root.General.Network.UPnP.NATTraversal.HTTPViewerPort=8090

root.General.Network.UPnP.NATTraversal.HTTPViewerPortSSL=8091

root.General.Network.UPnP.NATTraversal.HTTPViewerPort2=8070

root.General.Network.UPnP.NATTraversal.HTTPViewerPort2SSL=8071

root.General.Network.UPnP.NATTraversal.HTTPViewerPort3=8050

root.General.Network.UPnP.NATTraversal.HTTPViewerPort4=8030

Example: Set UPnP information

http://<servername>/cgi-bin/admin/param?action=update&General.Network.UPnP.Enabled=yes&General.Network.UPnP.NATTraversal.Enabled=no

Return:

OK

2.1.13	Get/Set PPPoE info

Example: Get PPPoE information

http://<servername>/cgi-bin/view/param?action=list&group=General.Network.PPPoE

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| Enabled | yes, no | 6/6 | Enables/disables PPPoE service. |
| IPAddress | IP address | 6/6 | IP address. The physical address of the device on the network. (read only) |
| UserID | none, string | 6/6 | PPPoE login user id. |
| Password | none, string | 6/6 | PPPoE login password. |
| DNSAuto | on, off | 6/6 | Specifies if the DNS server should be obtained from a DHCP server. |
| DNSServer1 | IP address | 6/6 | Primary Domain Name System server. |
| DNSServer | IP address | 6/6 | Secondary Domain Name System server. |

root.General.Network.PPPoE.Enabled=no

root.General.Network.PPPoE.IPAddress =67.34.251.69

root.General.Network.PPPoE.UserID=none

root.General.Network.PPPoE.Password=none

root.General.Network.PPPoE.DNSAuto=on

root.General.Network.PPPoE.DNSServer1=0.0.0.0

root.General.Network.PPPoE.DNSServer2=0.0.0.0

Example: Set PPPoE information

http://<servername>/cgi-bin/admin/param?action=update&General.Network.PPPoE.Enabled=yes&General.Network.PPPoE.UserID=example@hinet.net&General.Network.PPPoE.Password=12345678&General.Network.PPPoE.DNSAuto=on

Return:

OK

2.1.14	Get/Set DDNS info

Example: Get DDNS information

http://<servername>/cgi-bin/view/param?action=list&group=General.Network.DDNS

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| Enabled | yes, no | 6/6 | Enables/disables DDNS service. |
| DDNSServer | none, IP address, host name | 6/6 | The server that support DDNS service. |
| UserName | none, string | 6/6 | DDNS login user id. |
| Password | none, string | 6/6 | DDNS login password. |
| HostName | none, string | 6/6 | The name entered here will be associated with the product’s IP address in the DDNS server. |

root.General.Network.DDNS.Enabled=no

root.General.Network.DDNS.DDNSServer=www.dyndns.org

root.General.Network.DDNS.UserName=none

root.General.Network.DDNS.Password=none

root.General.Network.DDNS.HostName=none

Example: Set DDNS information

http://<servername>/cgi-bin/admin/param?action=update&General.Network.DDNS.Enable=yes&General.Network.DDNS.DDNSServer=www.dyndns.org&General.Network.DDNS.UserName=example&General.Network.DDNS.Password=12345678&General.Network.DDNS.HostName=www.example.dyndns.org

Return:

OK

2.1.15	Get/Set Time info

Example: Get Time information

http://<servername>/cgi-bin/view/param?action=list&group=General.Time

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| ObtainFromDHCP | yes, no | 6/6 | DHCP servers may provide names/IP addresses for local/remote NTP servers. Enable this feature by setting this parameter to yes. |
| ServerDate | yyyy-mm-dd | 6/6 | The date (yyyy-mm-dd) when the device's time was set manually or synchronized with the computer. |
| ServerTime | hh:mm:ss | 6/6 | The time (hh:mm:ss) when the device's time was set manually or synchronized with the computer. |
| DateFormat | ymd, mdy, dmy | 6/6 | Date format. |
| SyncSource | None, PC, NTP | 6/6 | The source to synchronize the time with; PC, NTP or None (manually). |
| TimeZone | GMT-12:00/Etc/GMT-12 GMT-11:00/Pacific/Apia GMT-10:00/Pacific/Honolulu GMT-09:00/America/Anchorage GMT-08:00/America/Los_Angeles GMT-07:00/America/Denver GMT-07:00/America/Tegucigalpa GMT-07:00/America/Phoenix GMT-06:00/America/Winnipeg GMT-06:00/America/Mexico_City GMT-06:00/America/Chicago GMT-06:00/America/Costa_Rica GMT-05:00/America/Indianapolis GMT-05:00/America/New_York GMT-05:00/America/Bogota GMT-04:00/America/Santiago GMT-04:00/America/Caracas GMT-04:00/America/Montreal GMT-03:30/America/St_Johns GMT-03:00/America/Thule GMT-03:00/America/Buenos_Aires GMT-03:00/America/Sao_Paulo GMT-02:00/Atlantic/South_Georgia GMT-01:00/Atlantic/Cape_Verde GMT-01:00/Atlantic/Azores GMT 00:00/Europe/Dublin GMT 00:00/Africa/Casablanca GMT 01:00/Europe/Amsterdam GMT 01:00/Europe/Belgrade GMT 01:00/Europe/Brussels GMT 01:00/Europe/Warsaw GMT 01:00/Africa/Lagos GMT 02:00/Europe/Athens GMT 02:00/Europe/Bucharest GMT 02:00/Africa/Cairo GMT 02:00/Africa/Harare GMT 02:00/Europe/Helsinki GMT 02:00/Asia/Jerusalem GMT 03:00/Asia/Baghdad GMT 03:00/Asia/Kuwait GMT 03:00/Europe/Moscow GMT 03:00/Africa/Nairobi GMT 03:30/Asia/Tehran GMT 04:00/Asia/Dubai GMT 04:00/Asia/Baku GMT 04:30/Asia/Kabul GMT 05:00/Asia/Yekaterinburg GMT 05:00/Asia/Karachi GMT 05:30/Asia/Calcutta GMT 05:45/Asia/Katmandu GMT 06:00/Asia/Almaty GMT 06:00/Asia/Dhaka GMT 06:00/Asia/Colombo GMT 06:30/Asia/Rangoon GMT 07:00/Asia/Bangkok GMT 07:00/Asia/Krasnoyarsk GMT 08:00/Asia/Hong_Kong GMT 08:00/Asia/Irkutsk GMT 08:00/Asia/Kuala_Lumpur GMT 08:00/Australia/Perth GMT 08:00/Asia/Taipei GMT 09:00/Asia/Tokyo GMT 09:00/Asia/Seoul GMT 09:00/Asia/Yakutsk GMT 09:30/Australia/Adelaide GMT 10:00/Australia/Brisbane GMT 10:00/Australia/Sydney GMT 10:00/Pacific/Guam GMT 10:00/Australia/Hobart GMT 10:00/Asia/Vladivostok GMT 11:00/Asia/Magadan GMT 12:00/Pacific/Auckland GMT 12:00/Pacific/Fiji GMT 13:00/Pacific/Tongatapu | 6/6 | Time zone. |
| UTCOffset | 00:00:00 | 6/6 | Time difference (hh:mm:ss) between the time zone and Coordinated Universal Time (UTC). -01:00:00 = -1 hour, i.e. 1 hour after UTC. 00:00:00 = 0 hour, 01:00:00 = 1 hour, i.e. before UTC. |

root.General.Time.ServerDate=1970-01-01

root.General.Time.ServerTime=02:30:14

root.General.Time.DateFormat=ymd

root.General.Time.SyncSource=NTP

root.General.Time.TimeZone=GMT 08:00/Asia/Taipei

root.General.Time.UTCOffset=00:00:00

Example: Set Time information

http://<servername>/cgi-bin/admin/param?action=update&General.Time.ObtainFromDHCP =no&General.Time.ServerDate=2007-09-18&General.Time.ServerTime=18:20:24&General.Time.DateFormat=ymd&General.Time.SyncSource=NTP&General.Time.TimeZone=GMT 08:00/Asia/Taipei&General.Time.UTCOffset=00:00:00

Return:

OK

2.1.16	Get/Set NTP info

Example: Get NTP information

http://<servername>/cgi-bin/view/param?action=list&group=General.Time.NTP

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| Server | IP address, host name | 6/6 | The NTP server to connect to when synchronizing the time in the device. |
| Update | 01:00:00 [time] | 6/6 | Time interval (hh:mm:ss) between connections to the NTP server. |
| VolatileServer | IP address, host name | 6/6 | The name/IP address of the NTP server, received from the DHCP server. Only one NTP server is currently supported. The NTP server name/IP address will be valid only until the next DHCP renewal or reboot. |

root.General.Time.NTP.Server=pool.ntp.org

root.General.Time.NTP.Update=01:00:00

root.General.Time.NTP.VolatileServer=0.0.0.0

Example: Set NTP information

http://<servername>/cgi-bin/admin/param?action=update&General.Time.NTP.Server=pool.ntp.org&General.Time.NTP.Update=01:00:00

Return:

OK

2.1.17	Get/Set Bonjour Info

Example: Get Bonjour information

http://<servername>/cgi-bin/admin/param?action=list&group=General.Network.Bonjour.Enabled

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| Enabled | yes, no | 6/6 | Enables/disables Bonjour service. |
| DeviceName | none or a string | 6/6 | The device name will be showed on the bonjour browser. If it uses the default value, this name will like the format below: [Brand]-[MacAddress]. |

root.General.Network.Bonjour.Enabled=yes

Example: Set Bonjour information

http://<servername>/cgi-bin/admin/param?action=update&General.Network.Bonjour.Enabled=yes&General.Network.Bonjour.DeviceName=Allnet-ipcam

Return:

OK

2.1.18	Get/Set Messenger Info

Example: Get Messenger information

http://<servername>/cgi-bin/admin/param?action=list&group=General.Network.Messenger

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| Enabled | yes, no | 6/6 | Enables/disables Messenger. |
| StartPort | Number | 6/6 | Lower bound port number for video tcp connection |
| EndPort | Number | 6/6 | Upper bound port number for video connection |
| NotifyIP | yes, no | 6/6 | Enables/disables notify IP while ipcamera account and users in AllowList are online. |
| Protocol | msn | 6/6 | Protocol type |
| Username | none or string | 6/6 | Messenger account |
| Password | none or string | 6/6 | Password |
| Nickname | none or string | 6/6 | Nickname shown in messenger |
| AllowList.User0 | none or string | 6/6 | User that is allowed to communicate with this Messenger account |
| AllowList.User1 | none or string | 6/6 | Same as above |
| AllowList.User2 | none or string | 6/6 | Same as above |
| AllowList.User3 | none or string | 6/6 | Same as above |
| AllowList.User4 | none or string | 6/6 | Same as above |

root.General.Network.Messenger.Enabled=on

root.General.Network.Messenger.StartPort=20000

root.General.Network.Messenger.EndPort=21000

root.General.Network.Messenger.NotifyIP=yes

root.General.Network.Messenger.Protocol=msn

root.General.Network.Messenger.Username=test8@hotmail.com

root.General.Network.Messenger.Password=ipcameratest

root.General.Network.Messenger.Nickname=ipcamera

root.General.Network.Messenger.AllowList.Enabled=yes

root.General.Network.Messenger.AllowList.User0=test-msn@hotmail.com

root.General.Network.Messenger.AllowList.User1=none

root.General.Network.Messenger.AllowList.User2=none

root.General.Network.Messenger.AllowList.User3=none

root.General.Network.Messenger.AllowList.User4=none

Example: Set Messenger information

http://<servername>/cgi-bin/admin/param?action=update&General.Network.Messenger.Enabled=yes&General.Network.Messenger.Username=test@hotmail.com&General.Network.Messenger.Password=ipcameratest

Return:

OK

2.2	Image/Video/Audio Category

2.2.1	  Get/Set http port number

Example: Get http port number

http://<servername>/cgi-bin/view/param?action=list&group=General.System

http://<servername>/cgi-bin/view/param?action=list&group=General.System.HTTPPort

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| HTTPPort | number | 1/6 | Http port number |

root.System.HTTPPort=80

Example: Set http port number

http://<servername>/cgi-bin/admin/param?action=update&General.System.HTTPPort =80

Return:

OK

2.2.2	  Get/Set video port number

Example: Get video port number

http://<servername>/cgi-bin/view/param?action=list&group=General.System

http://<servername>/cgi-bin/view/param?action=list&group=General.System.HTTPViewerPort

http://<servername>/cgi-bin/view/param?action=list&group=General.System.HTTPViewerPort2

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| HTTPViewerPort | number | 1/6 | MPEG4 viewer port number |
| HTTPViewerPort2 | number | 1/6 | MJPEG viewer port number |

root.System.HTTPViewerPort=8090

root.System.HTTPViewerPort2=8070

Example: Set http port number

http://<servername>/cgi-bin/admin/param?action=update&General.System.HTTPViewerPort=8090&General.System.HTTPViewerPort2=8070

Return:

OK

2.2.3	  Get/Set video resolution

Example: Get MPEG4 video resolution

http://<servername>/cgi-bin/view/param?action=list&group=Image.I0.Appearance.Resolution

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| Resolution | string | 1/6 | MPEG4 video resolution. |

root.Image.I0.Appearance.Resolution=704x480

Example: Get MJPEG video resolution

http://<servername>/cgi-bin/view/param?action=list&group=Image.I0.MJPEG.Resolution

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| Resolution | string | 1/6 | MJPEG video resolution. |

root.Image.I0.MJPEG.Resolution=704x480

Example: Set MPEG4 video resolution

http://<servername>/cgi-bin/admin/param?action=update&Image.I0.Appearance.Resolution=352x240

Return:

OK

Example: Set MJPEG video resolution

http://<servername>/cgi-bin/admin/param?action=update&Image.I0.MJPEG.Resolution=352x240

Return:

OK

2.2.4	  Get video sensor type

Example:

http://<servername>/cgi-bin/view/param?action=list&group=ImageSource.I0.SourceType

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| SourceType | NTSC, PAL, CMOS | 1/6 | Image source type. (read-only) |

root.ImageSource.I0.SourceType=NTSC

2.2.5	  Get/Set video parameters (Brightness, Contrast, Saturation, Hue)

Example: Get video parameters

http://<servername>/cgi-bin/view/param?action=list&group=ImageSource.I0.Sensor

Example: Set video parameters

http://<servername>/cgi-bin/admin/param?action=update&ImageSource.I0.Sensor.Brightness=50&ImageSource.I0.Sensor.Brightness=50&ImageSource.I0.Sensor.Contrast=50&ImageSource.I0.Sensor.Saturation=50&ImageSource.I0.Sensor.Hue=50

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| Brightness | 0~100 | 1/6 | Image brightness |
| Contrast | 0~100 | 1/6 | Image Contrast |
| Saturation | 0~100 | 1/6 | Image Saturation |
| Hue | 0~100 | 1/6 | Image Hue |

root.ImageSource.I0.Sensor.Brightness=50

root.ImageSource.I0.Sensor.Contrast=50

root.ImageSource.I0.Sensor.Saturation=50

root.ImageSource.I0.Sensor.Hue=50

2.2.6	  Get/Set video quality

Example: Get MPEG4 video quality

http://<servername>/cgi-bin/view/param?action=list&group=Image.I0.RateControl

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| MaxCompression | 1~31 | 1/6 | The rate controller will not produce streams with compression higher than this value. |
| MinCompression | 1~31 | 1/6 | The rate controller will not produce streams with compression lower than this value. |
| TargetBitrate | 2048, 1536, 1280, 1024, 768, 512, 384, 256, 128, 64 | 1/6 | The target bit rate, in kbit/s, for CBR mode. |
| Mode | auto, cbr, vbr | 1/6 | Specifies whether the rate controller operates in Variable Bit Rate (VBR) or constant bit rate (CBR) mode. |

root.Image.I0.RateControl.Mode=vbr

root.Image.I0.RateControl.TargetBitrate=2048

root.Image.I0.RateControl.MaxCompression=6

root.Image.I0.RateControl.MinCompression=6

Example: Set MPEG4 video quality (auto rate control)

http://<servername>/cgi-bin/admin/param?action=update&Image.I0.RateControl.MaxCompression=6&Image.I0.RateControl.MinCompression=6&Image.I0.RateControl.Mode=auto

Return:

OK

Example: Set MPEG4 video quality (fixed quality)

http://<servername>/cgi-bin/admin/param?action=update&Image.I0.RateControl.MaxCompression=1&Image.I0.RateControl.MinCompression=31&Image.I0.RateControl.Mode=vbr

Return:

OK

Example: Set MPEG4 video quality (constant bit rate)

http://<servername>/cgi-bin/admin/param?action=update&Image.I0.RateControl.MaxCompression=1&Image.I0.RateControl.MinCompression=31&Image.I0.RateControl.TargetBitrate=1024&Image.I0.RateControl.Mode=cbr

Return:

OK

Example: Get MJPEG video quality

http://<servername>/cgi-bin/view/param?action=list&group=Image.I0.Appearance.Compression

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| Compression | 0~100 | 1/6 | The level of image compression. High compression reduces the file size. Low compression produces optimum picture quality, but larger file sizes. 0: for auto rate control. |

root.Image.I0.Appearance.Compression=75

Example: Set MJPEG video quality (auto rate control)

http://<servername>/cgi-bin/admin/param?action=update& Image.I0.Appearance.Compression=0 Return:

OK

Example: Set MJPEG video quality (fix quality)

http://<servername>/cgi-bin/admin/param?action=update&Image.I0.Appearance.Compression=75

Return:

OK

Example: Set MJPEG video quality (fix bit rate)

Not support in MJPEG mode.

2.2.7	  Get/Set video frame rate

Example: Get MPEG4 video frame rate

http://<servername>/cgi-bin/view/param?action=list&group=Image.I0.Stream.FPS

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| FPS | 1~30 | 1/6 | Frame rate. Max value: 30 for NTSC/CMOS 25 for PAL. |

root.Image.I0.Stream.FPS=30

Example: Set MPEG4 video frame rate

http://<servername>/cgi-bin/admin/param?action=update&Image.I0.Stream.FPS=25

Return:

OK

Example: Get MJPEG video frame rate

http://<servername>/cgi-bin/view/param?action=list&group=Image.I0.MJPEG.FPS

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| FPS | 1~30 | 1/6 | Frame rate. Max value: 30 for NTSC/CMOS 25 for PAL. |

root.Image.I0.MJPEG.FPS=30

Example: Set MJPEG video frame rate

http://<servername>/cgi-bin/admin/param?action=update&Image.I0.MJPEG.FPS=25

Return:

OK

2.2.8	 Get/Set video flip state

Example: Get video flip state

http://<servername>/cgi-bin/view/param?action=list&group=Image.I0.Appearance.Flip

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| Flip | on, off | 1/6 | Enables/disables video Flip mode |

root.Image.I0.Appearance.Flip=off

Example: Set video flip state

http://<servername>/cgi-bin/admin/param?action=update&Image.I0.Appearance.Flip=yes

Return:

OK

2.2.9	 Get/Set video mirror state

Example: Get video flip state

http://<servername>/cgi-bin/view/param?action=list&group=Image.I0.Appearance.Mirror

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| Mirror | on, off | 1/6 | Enables/disables video Mirror mode |

root.Image.I0.Appearance.Mirror=off

Example: Set video flip state

http://<servername>/cgi-bin/admin/param?action=update&Image.I0.Appearance.Mirror=yes

Return:

OK

2.2.10	 Get/Set video night mode state (CMOS support only)

Example: Get video night mode

http://<servername>/cgi-bin/view/param?action=list&group=Image.I0.Appearance.NightMode

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| NightMode | auto, off | 1/6 | Enables/disables video Night mode |

root.Image.I0.Appearance.NightMode=off

Example: Set video night mode

http://<servername>/cgi-bin/admin/param?action=update&Image.I0.Appearance.NightMode=auto

Return:

OK

2.2.11	 Get/Set video light frequency (CMOS support only)

Example: Get video light frequency

http://<servername>/cgi-bin/view/param?action=list&group=Image.I0.Appearance.Lighting

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| Lighting | 50, 60 | 1/6 | Select video light frequency. |

root.Image.I0.Appearance.Lighting=off

Example: Set video frequency http://<servername>/cgi-bin/admin/param?action=update&Image.I0.Appearance.Lighting=50

Return:

OK

2.2.12	 Get/Set video white balance mode (CMOS support only)

Example: Get video white balance mode

http://<servername>/cgi-bin/view/param?action=list&group=Image.I0.Appearance.WhiteBalance

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| WhiteBalance | auto, fluorescent, incandescent, sunny, cloudy, blackwhite | 1/6 | Select video White Balance mode |

root.Image.I0.Appearance.WhiteBalance=auto

Example: Set video white balance mode

http://<servername>/cgi-bin/admin/param?action=update&Image.I0.Appearance.WhiteBalance=fluorescent

Return:

OK

2.2.13	 Get/Set video OSD

Example: Get http port number

http://<servername>/cgi-bin/view/param?action=list&group=Image.I0.OSD

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| Enabled | on, off | 1/6 | Enables/disables OSD. |
| Date | on, off | 1/6 | Enables/disables Date/Time OSD. |
| TextColor | white, black | 1/6 | OSD text color. |
| BkColor | white, black, transparent | 1/6 | OSD background color. |
| Position | Top, bottom | 1/6 | OSD position. |
| Alias | None, string | 1/6 | Alias for camera display on OSD. (must less than 6 characters) |

root.Image.I0.OSD.Enabled=off

root.Image.I0.OSD.Date=off

root.Image.I0.OSD.TextColor=white

root.Image.I0.OSD.BkColor=transparent

root.Image.I0.OSD.Position=top

root.Image.I0.OSD.Alias=None

Example: Set http port number

http://<servername>/cgi-bin/admin/param?action=update&Image.I0.OSD.Enabled=on&Image.I0.OSD.Date=on&Image.I0.OSD.TextColor=black&Image.I0.OSD.BkColor=transparent&Image.I0.OSD.Position=top&Image.I0.OSD.Alias=CAM1

Return:

Ok

2.2.14	 Get/Set video Mask

Example: Get http port number

http://<servername>/cgi-bin/view/param?action=list&group=Image.I0.Mask

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| Enabled | on, off | 1/6 | Enables/disables private mask. |
| XPos | number | 1/6 | The overlays upper left corner is positioned at this horizontal position (0=to the left). NTSC: 1~690 PAL : 1~690 COMS: 0~627 |
| YPos | number | 1/6 | The overlays upper left corner is positioned at this vertical position (0=to the top). NTSC: 1~234 PAL : 1~234 COMS: 0~471 |
| Width | number | 1/6 | Width of the specified mask. NTSC: 0~57 PAL : 0~57 COMS: 0~52 |
| Height | number | 1/6 | Height of the specified mask. NTSC: 0~29 PAL : 0~35 COMS: 0~59 |
| Color | black | 1/6 | The private mask color. |

root.Image.I0.Mask.Enabled=off

root.Image.I0.Mask.Date=off

root.Image.I0.Mask.TextColor=white

root.Image.I0.Mask.Width=transparent

root.Image.I0.Mask.Height=top

root.Image.I0.Mask.Color=None

Example: Set http port number

http://<servername>/cgi-bin/admin/param?action=update&Image.I0.Mask.Enabled=on&Image.I0.Mask.XPos=1&Image.I0.Mask.YPos=1&Image.I0.Mask.Width=20&Image.I0.Mask.Height=10&Image.I0.Mask.Color=black

Return:

Ok

2.2.15	 Get JPEG still image

Example:

http://<servername>/ jpg/image

Return:

Ok

http://<servername>/ jpg/image.jpg

Return:

HTTP/1.0 200 OK\r\n Content-Type: image/jpeg\r\n Content-Length: <image size>\r\n \r\n <JPEG image data>\r\n

2.2.16	Get video/audio stream

Example: Get MPEG4 video/audio stream (Only MPEG4 support audio frames)

http://<servername>/video.mp4

Return: MPEG4 video frames

HTTP/1.0 200 OK\r\n Content-Type: multipart/x-mixed-replace;boundary=<boundary>\r\n \r\n --<boundary>\r\n <image> where the proposed <boundary> is myboundary and the returned <image> field is below and please note that <sequence> is the sequence number of audio packets. Content-Type: video/mpeg\r\n Sequence: <sequence>\r\n Content-Length: <image size>\r\n \r\n <MPEG4 image data>\r\n --<boundary>\r\n <image>

Return: Mpeg4 audio frames

HTTP/1.0 200 OK\r\n Content-Type: multipart/x-mixed-replace;boundary=<boundary>\r\n \r\n --<boundary>\r\n <image> where the proposed <boundary> is myboundary and the returned <image> field is below and please note that <sequence> is the sequence number of audio packets. Content-Type: audio/basic\r\n Sequence: <sequence>\r\n Content-Length: <image size>\r\n \r\n <MPEG4 audio data>\r\n --<boundary>\r\n <image>

Example: Get MJPEG video stream

http://<servername>/video.mjpg

Return:

HTTP/1.0 200 OK\r\n Content-Type: multipart/x-mixed-replace;boundary=<boundary>\r\n \r\n --<boundary>\r\n <image> where the proposed <boundary> is myboundary and the returned <image> field is Content-Type: image/jpeg\r\n Content-Length: <image size>\r\n \r\n <JPEG image data>\r\n --<boundary>\r\n <image>

2.2.17	 Transmit audio stream data

Example:

http://<servername>/ in/view/transmit

2.3	Motion Detection Category

2.3.1	Get/Set motion parameters

Example: Get motion parameters

http://<servername>/cgi-bin/admin/param?action=list&group=Motion.M#

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| Enable | yes,no | 6/6 | Enable/disable this detection window. |
| Sensitivity | 0-512 | 6/6 | This tunes the "object difference from the background"-sensitivity, i.e. difference in terms of color and/or structure. A high value detects even very small changes and can e.g. trigger on image noise if set too high. A very low value requires on the other hand a very dramatic change with e.g. a dark object appearing in an almost white scene (or vice versa). |
| Threshold | 0-1000 | 6/6 | This adjusts the motion detection to be more or less sensitive for very fast motion. With a very low value the motion is detected only when a scene has changed but adapts to this changed scene as the new normal scene rapidly. A large value is more conservative and will detect the motion for a larger number of pictures before it consider this as being a normal object in the scene. |
| Left | 0-9999 | 6/6 | The coordinate for the left boundary of the rectangular motion detection window. The full value range is related to the full image width and 0 is counted from the left hand side of the image. |
| Top | 0-9999 | 6/6 | The coordinate for the upper boundary of the rectangular motion detection window. The full value range is related to the full image height and 0 is counted from the upper side of the image. |
| Right | 0-9999 | 6/6 | The coordinate for the right boundary of the rectangular motion detection window. The full value range is related to the full image width and 0 is counted from the left hand side of the image. |
| Bottom | 0-9999 | 6/6 | The coordinate for the bottom boundary of the rectangular motion detection window. The full value range is related to the full image height and 0 is counted from the upper side of the image. |

	Note: The # is replaced with a group number starting from zero, e.g. Motion.M0.

root.Motion.M0.Enabled=no

root.Motion.M0.Sensitivity=90

root.Motion.M0.Threshold=700

root.Motion.M0.Left=0

root.Motion.M0.Top=0

root.Motion.M0.Right=720

root.Motion.M0.Bottom=480

Example: Set http port number

http://<servername>/cgi-bin/admin/param?action=update&Motion.M0.Enabled=yes&IMotion.M0.Sensitivity=90&Motion.M0.Threshold=700&Motion.M0.Left=0&Motion.M0.Top=0&Motion.M0.Right=720&Motion.M0.Bottom=480

Return:

Ok

2.3.2	Get motion strength

Example:

http://<servername>:19999

Return:

| Parameter name | value | Security level (get/set) | Description |
| --- | --- | --- | --- |
| R1 | 0~1000 | 6/6 | The strength of motion region 1 |
| R2 | 0~1000 | 6/6 | The strength of motion region 2 |
| R3 | 0~1000 | 6/6 | The strength of motion region 3 |

R1=0,R2=0,R3=0   R1=0,R2=0,R3=0  R1=0,R2=0,R3=0 ...

3	SAMPLE VIDEO/AUDIO STREAM GET FLOW

- Decide video streaming format

MPEG4 or MJPEG.

Only MEPEG4 support audio stream.

- Decide device IP address and HTTP port number

Remote device IP address or host name.

Ex: “192.168.1.99” or “example.dyndns.org”

http port: 80

- Decide device access username and password

Use base64 encryption to encode username and password

Ex: “admin:admin”  to “YWRtaW46YWRtaW4=”.

- Get viewer port corresponding to video format.

Ex:

for MPEG4 video port

http://<servername>/cgi-bin/view/param?action=list&group=General.System.HTTPViewerPort

for MJPEG video port

http://<servername>/cgi-bin/view/param?action=list&group=General.System.HTTPViewerPort2

- Open socket (device IP: viewer port) to send video request command and receive video frames.

Ex: http://<servername>:<videoport>/video.mp4

http://<servername>:<videoport>/video.mjpg

Only MPEG4 support audio streaming. Use content type to distinguish between audio and video frames.

4	VC++ SAMPLE CODE for VIDEO/AUDIO STREAM

Following sample code is only for demonstrating how to get video/audio streaming.

a. Get Video/audio stream

Mpeg4ThreadProc()

{

//Open socket

CSocketHelper::Open(m_sIPAddr, m_Port);

if (!CSocketHelper::IsOk())

return FALSE;

//Send video request command

CString fmt, response="";

char uri[32];

if(m_CompressType==COMPRESS_MJPEG)

strcpy(uri, "/video.mjpg");

else

strcpy(uri, "/video.mp4");

//Add authentication

if(strlen(m_sBase64Auth) == 0)

fmt.Format("GET %s HTTP/1.0\r\n\r\n", uri);

else

fmt.Format("GET %s HTTP/1.0\r\nAuthorization: Basic %s\r\n\r\n", uri, m_sBase64Auth);

try {

SendStr(fmt);

ReadUntil(response, "\r\n\r\n");

//Main frames receive thread

while(1)

{

CString content_type = "";

do {

response = "";

//ReadUntil(response, "\r\n\r\n");

ReadUntilLFLF(response);

GetOptionAsString(response, "Content-Type:", content_type);

} while(strcmp(content_type, "image/jpeg")!=0 &&

strcmp(content_type, "video/mpeg")!=0 &&

strcmp(content_type, "audio/basic")!=0);

int recv_len;

GetOptionAsInt(response, "Content-Length:", recv_len);

Recv(m_ImageBuffer, recv_len);

BOOL isAudio = FALSE;

if(strcmp(content_type, "audio/basic")==0)

isAudio = TRUE;

if(m_VideoRecvFunc!=NULL)

(*m_ VideoRecvFunc)(m_ImageBuffer, recv_len, isAudio, m_VideoContext);

}

} catch (...) {

return 0;

}

return 0;

}

b. Transmit audio data

AudioDataTransmit(BYTE *pAudioBuf, DWORD size)

{

BYTE* pEncodeBuf = new BYTE[size];

int encodeSize = 0;

m_G711Encoder.encode(pAudioBuf, size, pEncodeBuf, encodeSize);

CSocketHelper socket;

socket.Open(m_hostIP, m_httpPort);

if (socket.IsOk())

{

NuString str, str1, str2;

str1.Format("--myboundary\r\nContent-Type: audio/wav\r\n");

str2.Format("\r\n--myboundary\r\n");

str.Format("POST /cgi-bin/view/transmit HTTP/1.0\r\nContent-Type: multipart/form-data; boundary=--myboundary\r\nContent-Length: %d\r\nAuthorization: Basic %s\r\n\r\n", str1.length()+encodeSize+str2.length(),g_Base64Auth);

socket.SendStr(str);

socket.SendStr(str1);

socket.Send(pEncodeBuf, encodeSize);

socket.SendStr(str2);

}

socket.Close();

delete pEncodeBuf;

}