Title:			RTL8139/810x Family Driver OEMSETUP DISK

History:			V5.00  --   07/18/2002

Name:			RTSOEM.500

Released By:		Vincent

Update reason from Oemdisk 400 to 500

| Driver Type | Old Ver | New Ver | Reason | Checked |
| --- | --- | --- | --- | --- |
| RSET8139 EXE | Ver501 | Ver503 | 1. Fix Loopback test failure during the cable disconnecting 2. Solve the network test with the P4 CPU. |  |
| MSLANMAN DOS | Ver323 | Ver323 | None Changed |  |
| MSLANMAN OS2 | Ver321 | Ver321 | None Changed |  |
| NDIS2DOS | Ver323 | Ver323 | None Changed |  |
| NDIS2OS2 | Ver321 | Ver321 | None Changed |  |
| NWCLIENT\DOS | Ver364 | Ver364 | None Changed |  |
| NWCLIENT\OS2 | Ver313 | Ver313 | None Changed |  |
| NWSERVER\42_5x | Ver329 | Ver501 | 1. New driver for RTL8139C+ 2. Provide CHSM1.11 SPEC. 3. Integrate C mode and C+ mode 4. Build the descriptor mode to improve the CPU utilization and the performance. 5. Add TCPIP checksum offload for Netware 5.1 6. When the RxOVW happen, the IMR value will change in ISR. 7. Solve the problems of the transmit overflow in the TestConsole. |  |
| NWSERVER\41x | Ver329 | Ver501 | Same as NWSERVER\42_5x |  |
| NWSERVER\40 | Ver329 | Ver501 | Same as NWSERVER\42_5x |  |
| NWSERVER\312 | Ver329 | Ver501 | Same as NWSERVER\42_5x |  |
| NWSERVER\311 | Ver329 | Ver329 | None Changed |  |
| CLIENT32 | Ver329 | Ver501 | Same as NWSERVER\42_5x |  |
| RTSPKT | Ver330 | Ver340 | 1. Fix the "Sharing IRQ" related issue when the users select one card by the bus_number and device_number. |  |
| SCO\5.x | Ver314 | Ver500 | 1.Write the descriptor mode to reach TX/RX function 2. Integrate C+ mode and C+ in a single driver |  |
| SCO\4.x | Ver314 | Ver314 | None Changed |  |
| WFW311 | Ver313 | Ver313 | None Changed |  |
| NT351 | Ver397 | Ver397 | None Changed |  |
| W95OSR2 | Ver397 | Ver397 | None Changed |  |
| WIN95A | Ver397 | Ver397 | None Changed |  |
| WIN98 | Ver397 | Ver504 | 1. RTL8139 C+ Ndis4/5/51 drivers first released. 2. Ndis5/51 support IEEE802.1p Packet Priority. 3. Ndis5/51 support Task Offload TCP_IP_CheckSum and TCP_LargeSend. 4. This Ndis4 driver can NOT work at Win95 OSR2. Win95a/OSR2 please using v.397 ndis3 driver. 5. These drivers could support CarBus device. 6. These drivers could support RTL8139/810X series adapter, too. But, RTL8139/810X series adapter DO NOT support 802.1p and Task Offload. 7. INF-file: C+ mode: Rename "802.1p Priority" to "802.1Q/1p VLAN Tagging". C/C+ : Removed "ChipOnCardbus" parameter for NDIS5/51. C/C+ : Replace a new HELP file for NDIS4 (rtsnthlp.hlp). 8. C+ mode implemented a new tuned-collision algorithm. 9. NDIS51 C+ mode support IEEE802.1Q Vlan Packet (Ieee8021QInfo). 10. NDIS5/51 C mode support CPU C3 mode. 11. NDIS5/51 C mode could recover phy stops issue (S4->Shutdown). 12. NDIS5/51 C mode compatible with C3 resume machine (timing issue). 13. NDIS5/51 C mode Link_Change_WakeUp depend on OS (CheckForHang needn't enable). 14. C mode: Couldn't force speed under S3/disable_WOL system issue. 15. Fixed the LDPS parameter at S3 system by the OEM customer requirement. 16. For HCT10 updating to QFE5 new testkit: 1c_KernelCall, 2c_WOLPattern and 2c_OffloadChecksum (C+ only) fail issues. 17. WinXP resume has to less than 5 secs issue 18. Win2000/XP BootROM remote install un-link issue 19. Set EarlyTxFIFO as '4' to fix CLKRUN incompability issue |  |
| WINNT | Ver397 | Ver504 | Same as Win98 |  |
| WINME | Ver397 | Ver504 | Same as Win98 |  |
| WIN2000 | Ver397 | Ver504 | Same as Win98 |  |
| WINXP | Ver397 | Ver504 | Same as Win98 |  |
| DMI | Ver1.10 | Ver1.10 | None Changed |  |
| Unix Ware 7 | Ver2.00 | Ver2.00 | None Changed |  |
| LINUX |  |  | Refer to : http://www.scyld.com/network/rtl8139.html |  |
| FREEBSD |  |  | Released |  |
| Mac OS | Ver3.00 | Ver3.10 | 1. When the users plug a rtl8139 cardbus card, Mac OS will show an item, such as "rtl8139", in the network configuration panel according to the cardbus CIS string. However, the users can not active rtl8139 by selecting that item. So our driver hides the item created by Mac OS and creates another item, such as "Ethernet rtl8139", and the users can active rtl8139 by selecting it. |  |
| Mac OS X |  | Ver111 | New |  |
| BROM |  | N/A | None Changed |  |
| RTOS |  | N/A | None Changed |  |
| WinDiag |  | Ver1.28 | 1: To Use WMI interface for information setting/gathering. 2: To install interrupt handler in diagnostics driver. 3: To design test plan for this program. 4: To add Magic Packet wakeup diagnostics function. 5: To support multiple adapter. |  |