Title:			RTL8139B/C(L) CardBus Driver OEM DISK

History:		V1.30  --   August/22/2001

Name:			RTSOEMCB.130

Released By:	Jean Chen

Update reason from Oemdisk 1.22 to 1.30

| Driver Type | Old Ver | New Ver | Reason |
| --- | --- | --- | --- |
| RSET8139 EXE | Ver501 | Ver501 | None Changed |
| NWCLIENT\RTSODI | Ver361 | Ver364 | (364) - Add a loop counter for early receive to prevent 8139 enter a wait next packet state. That will slow down the performance.. (363) - If 8139C enter loopback mode the receiver may dead on 90m cable and NS PHY. - In miniPCI and clock run enable the early transmit parameter should limit to 0x0a. (362) - Change the wait byte number of early interrupt from 4 to 64. - Add NONEARLY keyword to set the 8139 to run at non early interrupt mode. |
| NWCLIENT\RTPCI | Ver114 | Ver116 | (116) Use pci_set_word( ) to access Latency Timer for Toshiba (115) Add for Toshiba TOPIC 100. |
| RSET\RTPCI | Ver114 | Ver116 | Same above |
| RTSPKT\RTPCI | Ver114 | Ver116 | Same above |
| RTSPKT\RTSPKT | Ver321 | Ver330 | - Fix the bug about "SET_ADDRESS" call not function.Must enable EEM0,EEM1 to 1,1.1 - Fix the bug on "-n" command. |
| NDIS2DOS | Ver323 |  | None Changed |
| WIN95A | Ver378 | Ver397 | (397) 1. rtsnt.c: Fixed FTP download too slow issue. 2. power.c: a. NWay Foolproof Capability, that is to say, if user force 100Full and 100H/10F/10H could work normally. b. If user defined "WakeUp on ARP/PING- Disable", driver has to turn off the Unicast (offset 0xD8). c. When Shutdown, set/clear LinkUp (offset 0x59) by user defined of "WakeUp on Link Change - Enable/Disable". d. Add/Remove wakeup pattern depend on OS request OID. 3. link.c: Re-do Nway if Nway fail at first time. 4. pcb.c: Changed the access size of PCI Configuration Space Latency Timer Register from byte-access to word-access. 5. INF file: Rename "RTL8139 Family ..." to "RTL8139/810X Family ...". (395) - rtsnt.c: If our OEM engineer make a wrong SpeedDuplexMode parameter, we have to correct it. - power.c: We fixed following bug issue: When NIC from D1~D3 back to D0, if user define Wakeup Disable and force some speed mode. Our chip will back to auto-negotiation mode. (394) 1. INF files: - added Sub_Vendor and Sub_System ID for install (pass HCT use) - added "Receive Buffer Size" parameter - renamed "WakeUp on Link Status Change" to "WakeUp on Link hange" (since this parameter is to long for some Win98 PC) - removed "Smart Power Saving" parameter (analog power on/off depend on the decision of OS) 2. Added a new NDIS4 driver for Phoenix PowerPanel: This new NDIS4 driver could support Phoenix PowerPanel (for notebook). 3. CardBus: Completely support ALi chipset (392) 1. INF file: a) added "Smart Power Saving" parameter (analog power on/off) b) added "WakeUp on Link Status Change" parameter c) rename "WakeUp Frame" to "WakeUp on ARP/PING" d) rename "APM Mode WakeUp" to "WakeUp using APM Mode" 2. power.c: when system become sleeping mode, analog power will be OFF - Rename "Realtek RTL8139(A/B/C/8130) ." to "Realtek RTL8139 Family ..." (391) .(for Cardbus)Read MAC address directly from EEPROM. (390) - Remove Wakeup patterns when driver is removed(MiniportHalt). - Read MAC address directly from EEPROM. - Add 2 custom OID:OID_RTL8139_LINK_DUPLEX and OID_RTL8139_RESET_COUNTER. |
| WIN95B | Ver374 | Ver397 | Same above |
| WIN98 | Ver374 | Ver397 | Same above |
| WINNT4 | Ver374 | Ver397 | Same above |
| WIN2000 | Ver374 | Ver397 | Same above |
| WINXP |  | Ver397 | New |
| WinDiag\Win9x | Ver1.03 | Ver1.70 | (170) - Support the Duplex mode message in "Link Status" for RTL8139 NDIS4/NDIS5 driver version 390 and later. - Change "Reset" function from "ResetAdapter" to "OID_RTL8139_RESET_COUNTER" for RTL8139 NDIS4/NDIS5 driver version 390 and later. - Remove 1394NIC from Select dialog. (104) Add Target MAC Address Load and Save functions in Magic Packet Sender. Remove PPPMAC from Select dialog. |
| WinDiag\WINNT4 | Ver1.03 | Ver1.70 | (170) Same above (160) The diagnostic program can install the protocol driver automatically and the user does not have to install the protocol driver anymore before execute the diagnostic program. (104) Add Target MAC Address Load and Save functions in Magic Packet Sender. |
| WinDiag\WIN2000 | Ver1.03 | Ver1.70 | (170) Same above (150) The diagnostic program can install the protocol driver automatically and the user does not have to install the protocol driver anymore before execute the diagnostic program. (104) Same above |
| Linux | Ver1.11 |  |  |
| help8139.exe and \TXT | Ver1.10 |  | None Changed |
| Nwserver | Ver3.29 | Ver3.29 | None Changed |